COMBUSTION TECHNOLOGY

Aim of laboratory practice:

Investigation of gas-fired household combined heating and water heating equipment:

- Energy test of combustion apparatus (efficiency, flue gas loss)

- Environmental test of combustion apparatus (composition of flue gas)

In some sense, natural gas can be a renewable resource. A major plus of natural gas is that it can be efficiently and safely stored.

Natural gas is the cleanest burning fossil fuel. The combustion process for natural gas is almost perfect, very few byproducts are emitted into the atmosphere as pollutants. Also, with the introduction of new technologies, nitrogen oxide, a pollutant targeted by the Clean Air Act can be significantly reduced. The blue flame seen when natural gas is ignited is a sign of perfect combustion.

Natural gas is convenient. The energy source is piped directly to the customer's facility through the safe, efficient pipeline system. There's no need to store oil on site in tanks, or schedule oil deliveries.

Fig.1. Conceptual structure of household hot water and heating energy production equipment

Most cases, the heat transfer fluid is water. Favorable properties:

-	non-flammable,

-	high specific heat capacity,

-	cheap.

Types of combustion equipment: traditional and condensation. The condaensation equipments have higher efficiency (+condensation heat of water vapor from flue gas), but more expensive.

1. Basics of combustion technology:

The following processes take place during of methane combustion:

C + O2 + (7921 O2 number of moles) N2 = CO2 + (7921 O2 number of moles) N2

2H2 + O2 +(7921 O2 number of moles) N2 = 2H2O + (7921 O2 number of moles) N2

S + O2 + (7921 O2 number of moles) N2 = SO2 + (7921 O2 number of moles) N2

The combustion equipment usually has closed combustion chamber, so the flue gas loss is lower (~2%).

Misdirected process		energy loss and/or environmental pollution:

- soot: carcinogenic,

- carbon monoxide: toxic,

- nitrogen oxides: ground-level ozon      (toxic),

- hydrocarbons: greenhouse effect.

Solution: appropriate excees air factor and appropriate temperature of combustion chamber.

2. Combustion calculations:

2.1 Theoretical combustion air quantity (L0)

L0 (Nm3/Nm3) = necessary air quantity (Nm3) for complete combustion of 1 Nm3 fuel. If the natural gas contains only methane:

CH4 + 2O2 + (7921 O2 number of moles) N2 = CO2 + 2H2O + (7921 O2 number of moles) N2

2.2 Theoretical dry flue gas (V0d)

Theoretical wet flue gas contains carbon dioxid, sulfur dioxid, nitrogen.

V0d=CO2+SO2+N2Nm3Nm3

If the natural gas contains only methane:

V0d=CO2+N2Nm3Nm3

2.3 Theoretical wet flue gas (V0w)

Theoretical wet flue gas contains water vapor, carbon dioxid, sulfur dioxid, nitrogen.

V0w=CO2+SO2+H2O+N2Nm3Nm3

If the natural gas contains only methane:

V0w=CO2+H2O+N2Nm3Nm3

2.4 Excess air factor (n)

n=LL0Nm3Nm3Nm3Nm3		or simplified:

n=2121−O2measured		or more specifically:

n=1+V0dL0⋅O2measured21−O2measured

2.5 Actually produced (real) flue gas quantity (Vd, Vw)

Real flue gas quantity > Theoretical flue gas quantity

Reason: n

Real dry flue gas:

Vd=V0d+L0n−1 Nm3Nm3

Real wet flue gas:

Vw=V0w+L0n−1 Nm3Nm3

2.6 Boiler efficiency (thermic efficiency) (η)

Calculation of direct efficiency:

η%=G⋅cv⋅t2−t1Fé⋅Vg100

G – volume of boiler inlet water m3h

t2 és t1 – temperature of boiler outlet and inlet water oC

cv – specific heat of water   4186,8 kJm3   oC

Fé – heating value of gas 34000 kJNm3

Vg – volume of gas Nm3h

Calculation of indirect efficiency:

η=100−Qf+QCO+Qs

Qf – flue gas loss kJNm3

QCO – loss of imperfect combustion kJNm3

Qs – radiation loss kJNm3

2.7 Flue gas loss (Qf)

Qf=Vw⋅cf⋅(tf−tair)Fé⋅100%%

cf – heating value of  flue gas  1,35kJm3oC

tf  – temperature of flue gas oC

tair – temperature of inlet air oC

Fé – heating value of gas kJNm3

3. Laboratory equipment:

Fig.2. Schematic diagram of equipment

Combustion equipment: FERROLI Domitop F 24 E (24 kW)

- combined water heating and heating boiler

- traditional, not condensation boiler

- gas-fired

- closed combustion chamber

- with flue gas blower

- with coaxial chimney (flue gas + combustion air)

Flue gas analyser: TESTO 330

- exceed air factor: n (λ)

- efficiency: η [%]

- temperature of combustion air: Tlev

- temperature of flue gas: Tf

- concentration of CO, NOx

- O2%

Process of laboratory measurement:

- Open water and gas valve.

- Turn on (ON) combustion equipment.

- Open cooling water.

- Adjust the thermostat (30°C).

- Record values of gas and water meter, temperatures every 5 minutes.

- End of the measurement when inlet and outlet water temperature points on steady state level (min. 3 points)

- Analysis flue gas with TESTO 330 (O2, CO, NOx, n, η, Tlev, Tf)

- Finish measurement, turn off (OFF) combustion equipment, close water and gas tap.

Measuring diary:

- Short theory

- Table of measurement data

- Schematic diagram of equipment

- Calculations (Lo, Vod, Vow, n, Vd, Vw, η, Qf), results.

Measurement log

Date:								    Location: FII. 2.floor 5.		 Name:

| Heating system |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time |  | Gas meter [m3] | Gas pressure [mbar] | Temperature of combustion air [oC] | Temperature of flue gas [oC] |  | Composition of flue gas |  |  | Water meter [m3] | Temperature of water |  |
| [hour] | [min] |  |  |  |  |  | O2 [%] | CO [ppm] | NOx [ppm] |  | boiler inlet [oC] | boiler outlet [oC] |
| Pressure [mbar]: |  |  |  |  |  | Temperature of gas [oC]: |  |  |  |  |  |  |
| Termic efficiency (η, %): |  |  |  |  |  | n=λ: |  |  |  |  |  |  |